# §9 MCC / PMS Internal Architecture — Power System Reference

Document version: ARCH v4.0.1 supplement | Generated for CROSSBOW_CHANGELOG.md session entry

Scope: MCC §9.1 PMS power flow (all revisions) and §9.6 MCC_POWER enum reference. For full MCC architecture see ARCHITECTURE.md §9.

## 9.1 PMS Power Flow — Per Hardware Revision

The PMS (Power Management System) connects directly to the battery on all revisions. The MCC acts as the PDU (Power Distribution Unit), controlling power to all downstream accessories via GPIO-driven relays, solenoids, and Vicor enables. The compile-time defines HW_REV_Vx and LASER_xK together fully determine the pin map and power sequence — the runtime laser model sense from ipg.laserModel is a verification check only.

### V1 — 48V battery · 3kW laser

- 48V bat → PMS:

  - SOL_HEL (pin 5, HIGH=ON) → 3kW laser HV bus 48V

  - SOL_BDA (pin 8, HIGH=ON) → gimbal 48V. Sets isBDC_Ready.

  - TMC → 48V passthrough · no firmware switch

  - NTP appliance → 48V direct · no relay · no firmware control

- 48V bat → VICOR_BUS (A0, LOW=ON) → 24V relay bus:

  - RELAY_GPS (pin 83) → GPS appliance 24V

  - RELAY_LASER (pin 20) → 3kW laser digital bus 24V

### V2 — 300V battery · 6kW laser

- 300V bat → PMS:

  - VICOR_TMS (pin 20, HIGH=ON, NC opto) switched · 300V→48V → TMC

  - VICOR_GIM (A0, HIGH=ON, NC opto) switched · 300V→48V → gimbal. Sets isBDC_Ready.

  - GPS + NTP → unmanaged Vicor 300V→24V direct · no firmware switch

  - No relay bank on V2

- 300V bat → busbar direct → 6kW laser HV:

  - RELAY_LASER (pin 83, HIGH=ON) → 6kW laser digital enable

  - Note: pin 83 was RELAY_GPS on V1 and pin 20 was RELAY_LASER on V1 — functions swapped (the V2 hack)

### V3 — 48V battery · 3kW laser

- 48V bat → PMS (monolithic PCB):

  - SOL_HEL (pin 5, HIGH=ON) → 3kW laser HV bus 48V

  - SOL_BDA (pin 50, HIGH=ON) → gimbal 48V. Sets isBDC_Ready.

  - TMC → 48V passthrough via MCC board header · no firmware switch

- 48V bat → onboard VICOR_BUS (pin 40, HIGH=ON) → 24V relay bus:

  - RELAY_GPS (pin 67) → GPS appliance 24V

  - RELAY_NTP (pin 56) → NTP Phoenix appliance 24V (new on V3)

  - RELAY_LASER (pin 54) → 3kW laser digital bus 24V

  - Note: net label ARD_D40_LSR_PWR_EN is misleading — pin 40 is the relay bus Vicor enable, not a laser signal

### V3* — 300V battery · 6kW laser

- 300V bat → PMS:

  - VICOR_TMS (pin 51, HIGH=ON) switched · 300V→48V → MCC board power + TMC header passthrough

  - VICOR_GIM (pin 55, HIGH=ON) switched · 300V→48V → gimbal. Sets isBDC_Ready.

- VICOR_TMS 48V output → pickoff → onboard VICOR_BUS (pin 40):

  - VICOR_BUS PC pin held open at boot (always-on once VICOR_TMS is live) — no independent fw control

  - RELAY_GPS (pin 67) → GPS appliance 24V

  - RELAY_NTP (pin 56) → NTP Phoenix appliance 24V

  - RELAY_LASER not driven (pin 54 unused on 6kW path)

- 300V bat → busbar direct → 6kW laser HV:

  - PIN_ENERGIZE / RELAY_LASER bit 2 (pin 63, NO opto) closes → 5V out → external PSU enable

Note: V3* hardware is feasible with the V3 PCB by adding the external Vicor pack from V2 (VICOR_GIM + VICOR_TMS). VICOR_BUS receives its input from the VICOR_TMS 48V output — boot order dependency: VICOR_TMS must be enabled before relay bank is available.

### V4 — 48V or 300V battery · 3kW or 6kW laser (future)

- Consolidates V3 and V3* into a single voltage-variable platform.

- Solenoids (SOL_HEL, SOL_BDA) retired — bits 5 and 6 available for reassignment (e.g. LASER_HV_EN unified for both laser models).

- Board likely includes an input Vicor (bat→48V) to accommodate either battery voltage natively.

- RELAY_NTP and RELAY_GPS remain. Three-relay bank carried forward.

## 9.6 MCC_POWER Enum — Full Cross-Revision Reference

EnablePower(MCC_POWER p, bool en) is the sole function that calls digitalWrite on power output pins. All pin and polarity defines come from hw_rev.hpp. Outputs not valid for the active compile target are rejected in the default case with a debug print — no digitalWrite, no flag change.

POWER_BITS byte [10]: bit N = MCC_POWER enum value N. Unused outputs for active revision always pack 0 — no guards needed in the C# decoder. Read HW_REV byte [254] to confirm revision before interpreting V2-only bits (3, 4) vs V1/V3-only bits (0, 1, 5, 6).

| Bit | MCC_POWER enum | V1 48V · 3kW | V2 300V · 6kW | V3 48V bat · 3kW | V3* 300V bat · 6kW | V4 48V+300V · 3kW+6kW | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | compile: HW_REV_V1 LASER_3K | compile: HW_REV_V2 LASER_6K | compile: HW_REV_V3 LASER_3K | compile: HW_REV_V3 LASER_6K | compile: HW_REV_V4 |  |
| Relays — require VICOR_BUS first on V1 / V3 / V4 |  |  |  |  |  |  |  |
| 0 | RELAY_GPS | pin 83 NO HIGH=ON GPS 24V | — | pin 67 NO HIGH=ON GPS 24V | pin 67 NO HIGH=ON GPS 24V | TBD — | GPS appliance power V2: unmanaged Vicor 300V→24V direct — no relay |
| 2 | RELAY_LASER | pin 20 NO HIGH=ON 3kW digital bus 24V | pin 83 ⚠ pin reused from V1 RELAY_GPS 6kW digital enable | pin 54 NO HIGH=ON 3kW digital bus 24V | pin 63 PIN_ENERGIZE NO HIGH=ON ext PSU enable 5V out | TBD — | V2: same pin as V1 RELAY_GPS — the hack V3*: PIN_ENERGIZE NO opto closes → 5V out to ext PSU |
| 7 | RELAY_NTP | — direct powered no relay on V1 | — direct powered unmanaged Vicor | pin 56 NO HIGH=ON NTP 24V | pin 56 NO HIGH=ON NTP 24V | TBD — | Phoenix NTP appliance power New on V3 V1/V2: always powered |
| Vicors |  |  |  |  |  |  |  |
| 1 | VICOR_BUS | A0 LOW=ON 48V→24V relay bus | — no relay bank unmanaged Vicor 300V→24V direct | pin 40 HIGH=ON 48V→24V relay bus (net: ARD_D40 _LSR_PWR_EN — mislabelled) | pin 40 HIGH=ON PC always open Input from VICOR_TMS 48V pickoff | TBD — relay bus | Prerequisite for relay bank V3*: downstream of VICOR_TMS PC pin held open — no fw control Polarity flips V1(LOW=ON)→V3(HIGH=ON) |
| 3 | VICOR_GIM | — | A0 ⚠ pin reused from V1 VICOR_BUS NC HIGH=ON 300V→48V GIM | — 48V bat gimbal via SOL_BDA | pin 55 HIGH=ON 300V→48V gimbal supply | TBD — 300V→48V GIM | V2: same pin as V1 VICOR_BUS — the other half of the hack V3*: optional external Vicor pack identical function to V2 |
| 4 | VICOR_TMS | — | pin 20 ⚠ pin reused from V1 RELAY_LASER NC HIGH=ON 300V→48V TMS | — 48V bat TMC gets 48V passthrough no switch | pin 51 HIGH=ON 300V→48V MCC board + TMC passthrough PC always open at boot | TBD — 300V→48V TMS | V2: same pin as V1 RELAY_LASER V3*: feeds MCC board + TMC via header passthrough No fw switch in V3* (PC held open) |
| Solenoids — V1/V3 only · V4 retire; slots available for reassignment |  |  |  |  |  |  |  |
| 5 | SOL_HEL | pin 5 HIGH=ON 3kW laser HV bus 48V | — 6kW internal HV relay | pin 5 HIGH=ON 3kW laser HV bus 48V | — handled via PIN_ENERGIZE (RELAY_LASER bit 2) | TBD — → LASER_HV_EN? solenoid retired | V4: solenoid hardware removed bit 5 available for repurpose (e.g. LASER_HV_EN unified) |
| 6 | SOL_BDA | pin 8 HIGH=ON gimbal 48V | — VICOR_GIM powers gimbal | pin 50 HIGH=ON gimbal 48V | — VICOR_GIM powers gimbal | TBD — → free or repurposed | Sets isBDC_Ready on V1/V3-3kW V4: solenoid hardware removed |
| Laser model — compile-time define; runtime ipg.laserModel sense = verification only |  |  |  |  |  |  |  |
|  | hw_rev.hpp LASER_xK define | LASER_3K only error if LASER_6K | LASER_6K only error if LASER_3K | LASER_3K | LASER_6K | LASER_3K or LASER_6K | Runtime mismatch → fault log Does not change pin dispatch |
|  | Battery voltage | 48V | 300V | 48V | 300V | 48V or 300V | STM32 always sees 48V input |
|  | STM32 input | 48V direct | 300V→48V via VICOR_TMS/GIM (unmanaged) | 48V direct | 300V→48V via VICOR_TMS (PC open = always on) | TBD — likely external Vicor bat→48V |  |

## 9.7 Compile-Time Build Matrix

hw_rev.hpp enforces mutual exclusion across both axes at compile time. The laser model define gates pin dispatch in EnablePower() — no runtime branching on ipg.laserModel for power control.

| HW_REV | LASER_3K | LASER_6K | BATTERY | Valid? |
| --- | --- | --- | --- | --- |
| V1 | ✅ only | ❌ error | 48V only | ✅ |
| V2 | ❌ error | ✅ only | 300V only | ✅ |
| V3 | ✅ | ✅ | 48V or 300V | ✅ |
| V4 | ✅ | ✅ | 48V or 300V | TBD |

⚠ LASER_3K always implies 48V battery. LASER_6K always implies 300V battery (internal or external PSU). These are physically coupled — a mismatch is a hardware configuration error. The runtime ipg.laserModel sense provides fault detection only.